|  | ITEM NO. |
| --- | --- |

BRIEFING NOTE TO LEAD MEMBER FOR PLANNING

MAY 2006

TITLE :   	SUSTAINABLE URBAN DRAINAGE SYSTEMS (SUDS)

RECOMMENDATIONS :

EXECUTIVE SUMMARY :   THE LIABILITIES OF SUDS ON THE CITY COUNCIL AND   SEWERAGE UNDERTAKERS.

BACKGROUND DOCUMENTS :  N/A.

(Available for public inspection)

ASSESSMENT OF RISK:  N/A.

SOURCE OF FUNDING:  N/A.

COMMENTS OF THE STRATEGIC DIRECTOR OF CUSTOMER AND SUPPORT SERVICES (or his representative):

- LEGAL IMPLICATIONS				Provided by :  N/A.

- FINANCIAL IMPLICATIONS			Provided by :  N/A.

PROPERTY (if applicable):  Possible major impact on City Council Asset Management.

HUMAN RESOURCES (if applicable):  N/A.

CONTACT OFFICER :   Nigel Openshaw (Tel. No. 779 6110)

WARD(S) TO WHICH REPORT RELATE(S):  ALL.

KEY COUNCIL POLICIES:  TBA.

DETAILS (Continued Overleaf)

- BACKGROUND

1.1	Sustainable Urban Drainage Systems (SUDS) is a term used to describe methods of draining developments in a low cost, low energy, low maintenance and in a way with low environmental consequences.

1.2	The SUDS may incorporate methods of reducing discharges into the public sewerage and/or land drainage systems.

1.3	Approximately 4 years ago the Department of Environment Food and Rural Affairs (DEFRA) sent out a consultation seeking views on SUDS an officer response was subsequently sent, a draft SUDS procedure was subsequently produced.  The SUDS document, however, failed to deal with long term maintenance provision.

1.4	A further DEFRA consultation in 2003 – making space for water highlighted the need to reduce discharge into the drainage system.

- DEFRA

2.1	Of the 50,000 approximate no. of properties within the Salford area, over 6,000 lie within the River Irwell Flood Risk Area, a further 1,000 properties are at risk of internal flooding due to surcharge (overload) of the  public sewerage and land drainage systems.

2.2	Many other properties suffer from flooding in and around the properties during storms, this water however is in reality being stored until the storm and flows in the drainage systems subside and can be beneficial .

2.3	All the public sewerage systems, whether it be the surface water sewers which drain surface water from roads, car parks and roofs, or the combined sewerage system which drains both foul and surface water, are known to be at the limit of their capacity.

2.4	The local water courses are also known to be overloaded, therefore, discharge limits are placed on developments at pre planning (Development Team Approach (DTA) Stage, planning permission stage and Building Control Stage to ensure new developments do not cause flooding nuisances.  This approach has also been agreed with the Environment Agency and the Sewerage Undertaker United Utilities.

2.5	To limit discharge to sewers/watercourses, usually to the equivalent of a 150mm diameter pipe, developers are required to reduce the volume of surface water run off by considering the following measures:-

- Reduced impervious areas including porous paving.

- Incorporation of soakaways.

- Storage areas/ponds.

- Storage tanks.

- Over sized pipes and throttled outlet.

2.6	For larger developments it is important that the surface water run off is limited to the existing discharge from the site or to the Greenfield run off which has been agreed with UU and the EA to 7 L/S/Ha (7 litres/second/hectare).

2.7	To achieve a sustainable solution it is important that any large storage systems are constructed to adoptable standards to Sewers for Adoption 6th Edition and future maintenance liabilities transferred to United Utilities.

3.0	MAINTENANCE LIABILITIES

3.1	Drainage systems constructed to Sewers for Adoptions – i.e. clay and concrete pipes and manholes are designed to last a minimum of 80 years and require little maintenance, and would normally be adopted by United Utilities under an agreement - Section 104 of the Water Industry Act 1991. Future maintenance would be carried out by United Utilities.

3.2	Larger developments usually have roads adopted by the City Council under an agreement under Section 38 Highways Act 1980.

3.3	Large drainage systems utilising ponds and lagoons would not become adopted by United Utilities as they require significant maintenance such as dredging, fencing, cleaning, grass cutting, insuring and involves the transfer of land ownership. Sewerage systems are normally constructed under the adopted highway and do not require special access provision.

3.4	Large ponds and lagoons also require more frequent inspections and will require major maintenance after a few years such as that described in 3.3 above together with embankment reconstruction/repairs.

3.5	The City Council will not adopt such lagoons and ponds due to the funding of future maintenance/long term liabilities.  Adopted public sewerage systems are maintained by United Utilities.

3.6	Non adopted surface water drainage systems which do not discharge into the public sewerage systems are exempt from the surface water element of the water rates (usually approx. £100/property).

4.0	PLANNING/BUILDING REGULATIONS

4.1	Developers are advised of maximum discharge limits to public sewers and water courses at pre-planning, planning and Building Regulations stages, as agreed with United Utilities and where appropriate the Environment Agency.

4.2	Small developments of up to 10 operatives are advised to provide private sustainable drainage systems to limit the impact on the local drainage systems using the measures listed in paragraph 2.5 above (i.e. Ellesmere Park and Higher Broughton where there is a history of flooding the cellars, developments are restricted to 10 L/S).

4.3	Larger developments must have roads and sewers adopted by the City Council and United Utilities respectively to ensure long term maintenance safeguards for the property owners and the City Council/Sewerage Undertaker.

5.0	CONCLUSION

5.1	The above guidance has been given to developers for many years in order to safeguard the long term maintenance of developments.