ASSESSMENT FRAMEWORK

### Site Assessment

| Characteristic | Date and comments at 6 October 2004 |
| --- | --- |
| Location | Barton Lane, approximately 20m southwest of Oxford Street. A single two way, 2 lane (each approach) carriageway, total width 7.5 metres with 2.4, 2.5 metre footways. |
| Highway facilities | There are two street lighting columns in the close proximity. The road surface requires anti-skid treatment. |
| Visibility | Desirable visibility standards can be met. There are existing waiting restrictions (24 hours). |
| Complexity | There are road junctions, a public house and garage access in the close proximity. A primary school within 100 metres, However, the entrance/exit point to the school via side streets. |
| Crossing traffic | About 102 people cross the road from 7.30 - 9.30, 12.00 - 14.00, 15.00 - 18.00. Crossing time and difficulty of crossing are typical for roads of this character in the area. |
| Vehicles | 5037 vehicles average on a week day with 8% of heavy goods. Highest two way peak hour flow 714. Highest 85 percentile in peak periods is 33mph. There is a 30mph speed limit. |
| Road accidents | There were 9 personal injury accidents involving pedestrians, which resulted in 11casualties. - - 4 accidents - - 3 accidents - - 2 accidents |

### Option assessment

| Factor | Do nothing | Refuge island | Zebra | Signal Crossing |
| --- | --- | --- | --- | --- |
| Difficulty of crossing, average wait in seconds | 30 adults, 24 children & 2 elderly in peak (7.30 – 9.30 & 16.00 – 18.00) periods. | Not applicable due to carriageway width | Not recommended due to high 85th percentile speeds | 1 to 3 after end of vehicle minimum green period |
| Vehicle Delay in peak periods | None | N/A | N/A | 12 seconds |
| Road Capacity | Not reduced | N/A | N/A | 40% reduction |
| Representations |  | N/A | N/A | Individual letters from public in favour to meet safety needs of children, elderly and disabled people. AIU - pedestrian casualty savings. |
| Installation Cost | None at this stage | N/A | N/A | 25,000 |
| Operating Cost |  | N/A | N/A | 3,000 |