APPLICATION No:	04/49768/FUL

APPLICANT:	Rico Group Of Companies

LOCATION:	Riverside House Blackfriars Road Salford 3

PROPOSAL:	Erection of seven storey building to provide 104 apartments and below ground car parking together with associated amenities and construction of new and alterations to existing vehicular access

WARD:	Blackfriars

DESCRIPTION OF SITE AND PROPOSAL

Riverside House is currently vacant, although it has been used for offices. To the north of the site is the River Irwell and the associated river walkway. To the east is an area of trees, all of which are protected by a Tree Preservation Order, beyond which is a depot. To the north, the site is bounded by St Simon Street, beyond which are residential tower blocks and associated car parking. To the west is Blackfriars Road, beyond which is an area of vacant land.

It is proposed to erect a seven storey building comprising a total of 104 apartments. A total of 84 car parking spaces would be provided within the application site, the majority of which would be provided within a basement parking area. Vehicular access into the site would be provided from St Simon Street. There would be two main pedestrian entrances to the building, one at the corner of Blackfriars Road and St Simon Street and one at the corner of Blackfriars Street and the river walkway. The scheme would include the retention the protected trees on the land to the east of the proposed building and the three protected trees adjacent to the river walkway.

The scheme has been amended from that originally submitted and now includes pedestrian entrances into the ground floor flats fronting Blackfriars Road. The paved patio area between the building and river walkway would be private garden areas for the residents of the ground floor flats.

# SITE HISTORY

In February 2004, planning permission was granted for the erection of student accommodation comprising 240 bedrooms with ancillary accommodation including 15 car parking spaces together with associated creation of new vehicular access and alterations to existing vehicular access and landscaping (03/46609/FUL).

In November 2001, outline planning permission was granted for the use of the land for residential purposes (01/42911/OUT).

# CONSULTATIONS

## Director of Environmental Services – no comments received to date

## Environment Agency – objects to the application on the grounds that the building would be at risk of flooding and could unacceptably increase the risk of flooding elsewhere. The plans submitted do not show any floor levels to enable a full assessment in relation to flood risk to be made. Should additional information which clarifies the above issues be submitted, the Agency may be able to withdraw its objection

Police Architectural Liaison Officer – advises that the area is not one where liberal access to and from the site can be provided

Peak and Northern Footpath Society – no comments received to date

Ramblers’ Association – no objections

Greater Manchester Pedestrian Association – no comments received to date

Open Spaces Society – no comments received to date

Central Salford Urban Regeneration Company – no objections

PUBLICITY

The application has been advertised by both site and press notices

The following neighbour addresses were notified:

1-146 Blackfriars Court

1-12 Riverbank Lawns

## 1 St Simon Street

Friars County Primary School, Blackfriars Road

6 Blackfriars Road

REPRESENTATIONS

I have received three letters of objection in response to the planning application publicity.  The following issues have been raised:

Increase in noise

Increase in traffic

The development would result in the loss of an historic building

Insufficient parking

REGIONAL SPATIAL STRATEGY

## Site specific policies: none

## Other policies: none

UNITARY DEVELOPMENT PLAN POLICY

Site specific policies: CS2 – Greengate North

Other policies: 	H1 – Meeting Housing Needs

H6 – Open Space Provision within New Housing Developments

H11 – Open Space Provision within New Housing Developments

DEV1 – Development Criteria

DEV2 – Good Design

T13 – Car Parking

EN7 – Conservation of Trees and Woodland

# REVISED DEPOSIT DRAFT REPLACEMENT PLAN POLICY

Site specific policies: none

Other policies: 	H1 – Provision of New Housing Development

H8 – Open Space Provision Associated With New Housing Developments

DES1 – Respecting Context

DES6 – Waterside Development

DES7 – Amenity of Users and Neighbours

A1 – Transport Assessment and Travel Plans

A10 – Provision of Car, Cycle and Motorcycle Parking in New Developments

EN10 – Protected Trees

PLANNING APPRAISAL

The main planning issues associated with this application are: whether the principle of the proposal is acceptable; whether the design of the proposed building is acceptable; whether the proposed level of car parking is sufficient; whether there would be an appropriate contribution towards open space and environmental improvements in the Chapel Street Regeneration Strategy Area; and whether the proposal accords with the relevant policies of the Adopted and Revised Deposit Draft Replacement UDPs. I shall deal with each in turn below.

# Principle of the Proposal

Adopted Policy CS2 encourages the renewal of the Greengate North area through a number of measures, including the redevelopment vacant land between Trinity Way and the River Irwell for a mixture of housing, industry and non-retail commercial purposes and the improvement of pedestrian links to and along the River Irwell frontage.

Adopted Policy H1 states that the Council will endeavour to ensure that the city’s housing stock is able to meet the housing requirements of all groups within Salford by promoting a number of measures, including the release of land to accommodate new house building.

Draft Policy H1 states that new housing development should contribute to the provision of a balanced mix of dwellings within the local area.

Outline consent has already been granted for the redevelopment of the site for residential purposes. In addition, and more recently, full planning permission has been granted for the redevelopment of the site for student accommodation. I am of the opinion that both these permissions have established the principle of the use of the site for residential purposes. The proposal would make efficient use of a previously developed site in close proximity to the City Centre and a range of local facilities and services. A range of one and two bedroomed apartments would be proposed as part of this scheme. The application would contribute to the provision of a mix of dwellings in the area. I am therefore of the opinion that the principle of the proposal is acceptable and in accordance with Planning Policy Guidance Note 3: Housing, Adopted policies CS2 and H1 and Draft Policy H1.

# Design

Adopted Policy DEV2 states that planning permission will not normally be granted unless the Council is satisfied with the quality of the design and the appearance of the development.

Draft Policy DES1 of the Revised Deposit Draft Replacement UDP requires development to respond to its physical context, respect the positive character of the area and contribute to local identity and distinctiveness. In assessing the extent to which a proposal complies with this policy, regard will be had to a number of factors, including the relationship to existing buildings, the scale of the proposed development and the quality of the proposed materials.

The design of the proposed building is similar to the design of the student accommodation building approved on the site in February 2004. This building is however two storeys higher than that previously approved. The site is located in an area where there is no predominant building height which could be used to inform the height of this building. There are a number of high-rise apartment blocks in the vicinity, as well as two storey dwellings. I do not however have any objections to the height of the proposed building and consider its scale and massing to be appropriate. Given the similarities between this building and the building previously approved, I consider its design to be acceptable.

The application has been amended from that originally submitted and now includes pedestrian entrances to the ground floor flats fronting Blackfriars Street. I consider that this has improved the design and appearance on the proposed building and presents a more active frontage to the street, thereby increasing activity.

The applicant has indicated that brick and metal cladding would be used for the walls of the proposed building. I have attached a condition requiring samples of the materials to be submitted and approved prior to the commencement of the development. I am satisfied that this will ensure that the materials will be of a suitably high quality, in accordance with policies DEV1 and DES1.

# Car Parking

Adopted Policy T13 states that adequate and appropriate car parking should be provided to meet the needs of new development.

Draft Policy A1 states that planning applications for development which are likely to give rise to significant transport implications will not be permitted unless they are accompanied by a transport assessment.

Draft Policy A10 requires developments to make adequate provision for disabled drivers and cyclists and to not exceed the Council’s maximum car parking standards.

It is proposed to provide a total of 84 car parking spaces within the site. Planning Policy Guidance Note 13: Transport discourages Local Authorities from requiring developers to provide more parking than they themselves wish to provide, other than in circumstances where there would be significant implications for road safety which cannot be resolved through the introduction or enforcement of on-street parking controls. In line with this national guidance, Policy A10 advocates the use of maximum car parking standards. Secure cycle parking facilities would also be provided within the site which would also accord with this policy and would encourage the use of non-car modes of transport. The site is also located in close proximity to public transport links. In light of the above policy framework and in view of the non-car alternatives available, I consider the number of spaces provided to be acceptable.

In accordance with Policy A1, the applicant has submitted a Transport Assessment (TA) in order to assess the traffic implications of the proposed development. This concludes that the proposed development would not have a detrimental impact on the adjacent highway network. Whilst I acknowledge that an increase in traffic as a result of this proposal is inevitable, I do not consider that this increase would be so significant so as to have an unacceptable impact on the highway network. I agree with the conclusion of the applicant’s TA and am satisfied that highway safety would not be unacceptably compromised as a result of this application.

Contributions Towards Open Space and Environmental Improvement in Chapel Street

Adopted Policy H6 requires adequate provision of informal open space and children’s play within new housing developments. This policy is linked to Policy H11 which sets out a sliding scale for such provision.

Draft Policy H8 requires adequate and appropriate provision to be made for formal and informal open space within housing developments.

The Chapel Street Development Control Policy Note requires applicants for developments within the Chapel Street Regeneration Strategy Area to make a contribution towards environmental improvements within the area. For residential schemes, this equates to £1,000 per apartment.

In accordance with the above policies, the applicant has agreed to make a contribution towards the provision and maintenance of open space in the vicinity and environmental improvements within the Chapel Street area. In accordance with the recently adopted Supplementary Planning Guidance on open space, the contribution in this regard would be £147,184, with £104,000 towards environmental improvements. I have attached a condition requiring such a contribution. I am therefore satisfied that the application therefore accords with Adopted policies H6 and H11, Draft Policy H8 and the Chapel Street Development Control Policy Note.

# Other Issues

Objectors have raised concerns relating to an increase in noise as a result of this application. Whilst there may be an increase in noise in the area during construction, this is an inevitable part of development and is not in my opinion sufficient to warrant refusal of the application. Given that the proposal is for residential development, I therefore have no objections to the application in this regard.

One objector has also raised concerns regarding the loss of a historic building. Riverside House is not listed. In light of this, and given the previous approvals, I have no objections to the loss of the building.

The application would necessitate the closure of Watkin Street. I have notified the relevant bodies and have not received any objections to the application in this regard. I have however attached a condition requiring the appropriate order for the closure or diversion of the right of way to be obtained prior to the commencement of the development.

I have received an objection to the application from the Environment Agency. The Agency is concerned that the proposed building would be at risk from flooding and in the absence of level information, the impact in relation to flood risk cannot be fully assessed. The applicant is currently in the processes of amending the scheme to take the Agency’s concerns into account. In light of the fact that, following amendments, the Agency had no objections to the scheme for student accommodation previously approved on this site, I am confident that this proposal can be amended and that the Agency’s concerns can be overcome.

VALUE ADDED TO DEVELOPMENT

In accordance policies H6 and H11 of the Adopted UDP and the Chapel Street Development Control Policy Note, the applicant is required to contribute £147,184 and £104,000 respectively. This would contribute to the provision of open space in the vicinity and environmental improvements in the Chapel Street area.

The scheme has been amended from that originally submitted to include pedestrian entrances to the apartments fronting Blackfriars Road. This has improved the appearance of the building and will encourage greater activity in the area.

# CONCLUSION

In conclusion, I consider the application to be acceptable. It would result in the redevelopment of a brownfield site in an accessible location and complies with the provisions of the relevant policies of both the Adopted and Revised Deposit Draft Replacement UDPs. I therefore recommend that permission be granted.

RECOMMENDATION:

Approve Subject to the following Conditions

1.	Standard Condition A01 Five year time limit

2.	Prior to the commencement of the development hereby approved, samples of the materials for the external elevations and roof of the building shall be submitted to and approved in writing by the Local Planning Authority. The scheme shall be carried out using the approved materials, unless agreed otherwise in writing by the Local Planning Authority.

3.	The site shall be treated in accordance with a landscape scheme which shall be submitted to and approved by the Local Planning Authority before development is started.  Such scheme shall include full details of trees and shrubs to be planted, walls, fences, boundary and surface treatment and shall be carried out within 12 months of the commencement of development and thereafter shall be maintained to the satisfaction of the Local Planning Authority.  Any trees or shrubs dying within five years of planting shall be replaced to the satisfaction of the Local Planning Authority

4.	No development shall be started until the appropriate orders for the closure or diversion of the right of way have been approved.

5.	The windows of all habitable rooms to the elevation facing Blackfriars Road shall be double glazed units comprising 10mm / 12mm / 6mm - glass / air / glass prior to first occupation of any of the units hereby approved, and shall be retained at all times the units are occupied.  These shall be situated in accordance with the manufacturers recommendations to avoid air gaps when fitting the frames.  All habitable rooms to the elevation facing Blackfriars Road and the bedrooms to the elevations of St Simon Street and the River Irwell shall be provided with mechanical ventilation to the standard of the Noise Insulation Regulations 1975 (as amended) prior to first occupation of any of the units hereby approved, and shall be retained at all times the units are occupied.

6.	Prior to the commencement of the development, the developer shall submit a site investigation report for the approval of the Local Planning Authority.  The investigation shall address the nature, degree and distribution of ground contamination and ground gases on site and shall include an identification and assessment of the risk to receptors as defined under the Environmental Protection Act 1990, Part IIA, focusing primarily on risks to human health and controlled waters.  The investigation shall also address the implications of ground conditions on the health and safety of site workers, on nearby occupied building structures, on services and landscaping schemes and on wider environmental receptors including ecological systems and property.

The sampling and analytical strategy shall be approved by the Local Planning Authority prior to the start of the site investigation survey.  Recommendations and remedial works contained within the approved report shall be implemented by the developer prior to occupation of the site.

Prior to discharge of the Contaminated Land Condition, a Site Completion Report shall be submitted to the Local Planning Authority for approval.  The Site Completion Report shall validate that all works undertaken on site were completed in accordance with those agreed by the Local Planning Authority.

7.	Standard Condition C04X Fencing of Trees protected by T.P.O.

8.	The development permitted by this planning permission shall not be started by the undertaking of a material operation as defined in Section 56(4) (a-d) of the Town and Country Planning Act, 1990 until a Planning Obligation under Section 106 of the Town and Country Planning Act, 1990 has been made and lodged with the Local Planning Authority, and the Local Planning Authority has given its approval in writing. The planning obligation will provide that a commuted sum as required by policies H6 and H11 of the City of Salford Adopted UDP, Policy H8 of the Revised Deposit Draft Replacement UDP and SPG7 Provision of Open Space and Recreation Space Associated with New Residential Development and by the Salford City Council Development Control Policy Note - The Use of Planning Obligations in the Chapel Street Area will be paid to the Local Planning Authority for open space and recreation space purposes.

9.	The development hereby approved shall not be brought into use until the means of vehicular access from St Simon Street has been constructed and laid out in accordance with the approved plans.

10.	The car parking spaces shall be laid out in accordance with the approved plans prior to first occupation of any of the apartments hereby approved and shall be made available for use by residents at all times the premises are in use.

11.	Prior to the commencement of the development hereby approved, a scheme showing the provision of waste recycling facilities within the site shall be submitted to and approved in writing by the Local Planning Authority. The approved scheme shall be implemented prior to first occupation of any of the apartments hereby approved, unless agreed otherwise in writing by the Local Planning Authority.

12.	Prior to the commencement of the development hereby approved, a scheme showing the finished floor levels of the building to be not less than 28.3m AOD shall be submitted to and approved in writing by the Local Planning Authority. The development shall be carried out in accordance with the approved scheme.

(Reasons)

1.	Standard Reason R000 Section 91

2.	Standard Reason R004A Amenity-area

3.	Standard Reason R004A Amenity-area

4.	Standard Reason R026A Interests of highway safety

5.	Standard Reason R024A Amenity of future residents

6.	Standard Reason R024A Amenity of future residents

7.	Standard Reason R010A Protect TPO trees

8.	Reason: To ensure the residential development provides appropriate open space and recreation space for future occupiers anad contributes to environmental improvements within the Chapel Street Regeneration Strategy Area, in accordance with policies H6 and H11 of the City of Salford Adopted UDP, Policy H8 of the Revised Deposit Draft Replacement UDP, SPG7 Provision of Open Space and Recreation Space Associated with New Residential Development and Salford City Council Development Control Policy Note - The Use of Planning Obligations in the Chapel Street Area.

9.	Standard Reason R026A Interests of highway safety

10.	Standard Reason R012A Parking only within curtilage

11.	Reason: In order to encourage waste recycling, in accordance with Policy MW11 of the Adopted City of Salford Unitary Development Plan

12.	In order to reduce the risk of flooding

## Note(s) for Applicant

1.	The proposed development lies within a coal mining area. In the circumstances the Applicant should take account of any coal mining related hazards to the stability of their proposal. Developers must also seek permission from the Coal Authority before undertaking any operation that involves entry into any coal or mines of coal, including coal mine shafts and adits and the implementation of site investigations or other works.   Property specific summary information on any past, current or proposed surface and underground mining activity to affect the development can be obtained from the Coal Authority. The Coal Authority Mining Reports Service can be contacted on 0845 762 6848 or at www.coal.gov.uk

2.	The applicant is advised that the requirements of all the conditions precedent must be satisfied prior to the commencement of the development. Failure to satisfy the conditions precedent renders all development unauthorised and unlawful and appropriate action may be taken by the Council.

3.	The applicant is advised to contact the Council's Highways Maintenance Section regarding the making good of the redundant footway crossings.

4.	The applicant is advised that the basement of the building may be vulnerable to flooding.

5.	For further discussions regarding the requirements of the Contaminated Land Condition, the applicant is advised to contact the Council's Environmental Protection Team.

APPLICATION No:	04/49760/COU

APPLICANT:	CCS (Skip Hire) Limited

LOCATION:	Jones Executive Coaches Ltd Old Wellington Road Eccles

PROPOSAL:	Change of use of coach/bus depot to waste transfer station

WARD:	Eccles

DESCRIPTION OF SITE AND PROPOSAL

This application relates to a site within the Lyntown Trading Estate. Planning permission is sought for a change of use from a coach depot to a waste transfer station.

There are two existing buildings at the site – a garage and a single storey office building, in addition to a steel container. All waste operations, that is, bulking, storage, segregation and screening would be undertaken within the existing garage building. The existing steel container would be used as a quarantine area for items that should not be dealt with at the site, such as fridges and gas canisters. The existing building would provide an office, canteen and toilet facilities. Two parking spaces and a turning area are proposed in the yard. Empty skip storage, up to 2.1 metres in height is proposed along the northern site boundary, adjacent to the M602 motorway. A skip to store ferrous metal is proposed to be located adjacent to the garage building.

The proposed hours of operation are 07:30 – 18:00 Monday to Saturday and 10:00 – 16:00 Sundays and Bank Holidays.

It is proposed that one HGV will be used, which will make between 5 and 6 round trip journeys per day.

To the north of the site is the M602 motorway, beyond which there are residential properties. On either side and opposite the application site are industrial/commercial uses.

# SITE HISTORY

98/38656/FUL - Erection of two bay garage. Approved 21.12.98

# CONSULTATIONS

Director of Environmental Services – There may be an increase in traffic passing through the Russell Street residential area to gain access to the site. The surface of the yard is potholed and may require upgrading to ensure proper drainage and cleaning. The nearest dwellings are to the north of the M602, but there is an uninterrupted view of the development site and may be affected by noise in the evenings/weekend, for this reason, it is recommended that hours of operation are restricted. The following conditions are recommended: all waste transfer operations shall take place inside the building and there shall be no storage of wastes outside; there shall be no burning of materials; hours of working should be reduced to 7.30am-6pm (Monday to Friday) and 8am-3pm Saturdays, with no working on Sundays and Bank Holidays.

Environment Agency – No comments received

Highways Agency –  No objections in principle. Recommend conditions that there shall be no development on or adjacent to the motorway embankment that shall put any such embankment or earthworks at risk and that no drainage from the proposed development shall run off into the motorway drainage system, nor shall any such new development adversely affect any motorway drainage.

Greater Manchester Geological Unit – Detailed comments provided. Site location would seem appropriate for this type of development. Asked for clarification on a number of matters including a detailed description of the proposed operations and confirmation of vehicle movements. Proposed operation will require a waste management licence to be issued by the Environment Agency.

Greater Manchester Waste Disposal Agency –  No objections.

PUBLICITY

A press notice was published 20th January 2005

A site notice was displayed on 21st January 2005

The following neighbour addresses were notified:

Massey Coldbeck Ltd., Old Wellington Road

Tawny Ltd., Old Wellington Road

Ritrama Ltd., Lynwell Road

REPRESENTATIONS

I have received two letters of representation / objection in response to the planning application publicity.  The following issues have been raised:-

- concern that the applicant has no way of knowing what is included in domestic waste (i.e. food waste) and there will be an almost inevitable risk of seepage and an attraction to vermin. There could be a serious health risk as the company objecting manufactures products for the food industry.

- Also concerned about offensive odours from rotting foodstuffs

- Concerns raised about volume of traffic and ability of neighbouring company to access their premises

- A letter of objection has also been received from the neighbouring business who were located next to a site previously operated by CCS at The Oasis in Irlam. They are concerned that the previous use caused a constant nuisance due to their illegal tipping and burning of waste. The concern is that burning waste will cause a hazard to drivers on the motorway.

# REGIONAL SPATIAL STRATEGY

## EQ5 A Regional Approach to Waste Management

UNITARY DEVELOPMENT PLAN POLICY

Site specific policies: None

Other policies: 	MW11 - Waste Recycling and Bulk Reduction

MW15 - Development Control – Waste Criteria

DEV1 - Development Criteria

T13 - Car Parking

EN20 - Pollution Control

# REVISED DEPOSIT DRAFT REPLACEMENT PLAN POLICY

## Site specific policies: None

Other policies: 	W1 - Waste Management

DES1 - Respecting Context

DES7 - Amenity of Users and Neighbours

A10 - Provision of Car, Cycle and Motorcycle Parking in New Developments

EN14 - Pollution Control

PLANNING APPRAISAL

The main planning issues relating to this application are the principle of the use, the impact on residential amenity, the impact on visual amenity and the impact on the pollution of the environment and associated nuisance to neighbouring occupiers.

RSS policy EQ5 states that there should be increased re-use and recycling of household, commercial and industrial waste. Policies MW11 and MW15 encourage proposals that recycle waste and conserve resources unless visual or residential amenity is unduly affected or the proposal would be unacceptable in terms of access, traffic generation and road safety. Policy W1 supports the same principles and aims of the above policies. The proposal to operate a waste transfer station in this location and to sort material to be recycled is supported in accordance with the above policies.

With regards to impact on residential amenity, the closest dwellings are located on the opposite side of the M602 motorway and are at a minimum distance of 60 metres. There is existing 2.4 metre high galvanised steel palisade security fencing on the site boundaries, including the boundary with the M602 embankment. There are also a number of trees at the top of the motorway embankment which provide some screening. The majority of activity at the site will be within the building, but there will be some noise expected from the yard area, for example, the stacking of empty skips. There are also dwellings located on Russell Street and Chadwick Road which are the roads providing vehicular access to the site and as such, the impact of vehicle movements should be considered. I do not consider that the level of proposed vehicle movements would cause any significant detrimental impact on amenity over and above that already experienced by these residents, considering the existing use of the site as a coach and surrounding commercial uses within the trading estate. The Director of Environmental Services is concerned that there may be a detrimental impact on the amenity of residents on the opposite side of the M602, outside of times when the motorway is busy. For this reason, it is proposed that the hours of operation are restricted to no working Sundays and bank holidays. In terms of visual amenity, a condition is recommended to ensure that there is no storage of empty skips above 2.1 metres in height. This is below the height of the existing fence and will minimise visual impact.

Policy EN20 of the Adopted UDP and policy EN14 of the Replacement Plan seek to prevent harm to the environment that may be caused by development proposals. The Director of Environmental Services has no objections to the proposal, subject to a number of conditions. The Director of Environmental Services has advised that there is unlikely to be any dust arising from the proposed development as the operations will be carried out within the building. With regards to the objection raised in relation to burning of waste, a condition is recommended to prevent any burning at the site.

With regards to the objections raised in relation to the type of waste, seepage, generation of smells and attraction of vermin, the applicant has provided a supporting statement which details the type of waste which will be accepted at the site. No liquids will be accepted on the site, unless in retail packaging and general biodegradable waste, including household waste, will only be accepted as an inherent component of skip waste. All incoming waste will be deposited within the building and will then be processed. Non-ferrous general waste will be bulked up in a skip within the building and taken off site for disposal. Ferrous waste will be placed within a skip to be positioned between the building and the M602 site boundary. An existing lockable steel container structure within the yard will be used to store quarantined waste. A condition is recommended to ensure that there is no outside storage, processing or stockpiling of waste within the curtilage of the site, with the exception of storage within the container identified for quarantine, storage of empty skips and the siting and use of a ferrous metal skip, in the respective locations shown on the submitted plans.

# CONCLUSION

The proposal would contribute to sustainable waste management advocated in policies MW11, MW15 and W1, which seek to encourage higher recycling rates subject to the protection of the environment, the amenity of neighbouring occupiers and the compatibility of the site with surrounding land uses.

The site is situated in an existing industrial area and is some distance from residential areas, thereby achieving minimal impact on the local population and the environment. The proposal would employ two staff, bring a vacant site within an industrial estate into beneficial use and is consistent with PPG10, being sited in an ideal location consistent with polices MW11, MW15 and W1. I have no highway or drainage objections and recommend approval.

RECOMMENDATION:

Approve Subject to the following Conditions

1.	Standard Condition A01 Five year time limit

2.	This permission shall relate to the amended plan and supporting statement received on 21st February 2005 which shows the location of the quarantine area, empty skip storage area and ferrous metal skip.

3.	This permission shall relate to the use of the existing garage building at the site for the importation, storage, sorting and recycling of the materials identified in the submitted Working Statement received 21st February 2005 only.

4.	The height of all empty skips, subject of this planning permission, shall not exceed 2.1 metres above the existing ground level of the site at any time. The empty skips shall only be stored in the area indicated on the plan submitted 21st February 2005.

5.	Prior to any part of the site first becoming operational, a vehicle turning circle and two car parking spaces shall be marked out and hardsurfaced prior to the first use hereby approved and made available thereafter at all times the premises are in use.

6.	There shall be no burning on the site at any time.

7.	There shall be no outside storage, processing or stockpiling of waste within the curtilage of the site, with the exception of storage within the container identified for quarantine, the storage of empty skips and the siting and use of a ferrous metal skip, in the respective locations shown on the submitted plan dated 21st February 2005.

8.	The use hereby permitted shall NOT be operated on Sundays and Bank Holidays and shall ONLY be operated between the hours of 7.30am and 6.00pm on Mondays to Fridays and 8.00am to 3.00pm on Saturdays.

9.	The maximum tonnage of all material stockpile areas, subject of this permission, shall not exceed 1000 tonnes as measured over a period of 1 month at any time and 4, 999 tonnes over any 12 month period.

(Reasons)

1.	Standard Reason R000 Section 91

2.	Standard Reason R004A Amenity-area

3.	Standard Reason R004A Amenity-area

4.	Standard Reason R004A Amenity-area

5.	Standard Reason R011A Parking, turning etc within curtilage

6.	Standard Reason R004A Amenity-area

7.	Standard Reason R004A Amenity-area

8.	Reason: In the interests of the amenity and quietitude of the occupiers of nearby residential properties in accordance with policy DEV1 of the City of Salford Unitary Development Plan.

9.	For the avoidance of doubt and in accordance with policies MW11 and MW15 of the City of Salford Adopted Unitary Development Plan and policy W1 of the City of Salford Revised Deposit Draft Replacement Unitary Development Plan.

## Note(s) for Applicant

1.	It is recommended that any surface water drainage is fitted with a silt trap prior to connection to main drainage outfall to prevent blockages/silt/materials entering the sewer.

2.	The applicant's attention is drawn to the contents of the attached letter from the Greater Manchester Geological Unit

3.	The applicant's attention is drawn to the contents of the attached letter from the Highways Agency

4.	There shall be no development on or adjacent to the motorway embankment that shall put any such embankment or earthworks at risk and that no drainage from the proposed development shall run off into the motorway drainage system, nor shall any such new development adversely affect any motorway drainage.

APPLICATION No:	04/49762/FUL

APPLICANT:	Ania Aburomia

LOCATION:	Garden At Rear Of  53 Cavendish Road Eccles M30 9EE

PROPOSAL:	Sub-division of residential curtilage, demolition of existing garage and erection of two storey detached dwelling together with creation of new vehicular access

WARD:	Eccles

DESCRIPTION OF SITE AND PROPOSAL

This application relates to the rear garden of 53 Cavendish Road, Eccles.  The surrounding area is predominantly residential in character.  Adjacent to the southeast is  53 Cavendish Road.  Adjacent to the northeast is 55 Cavendish Road.  Adjacent to the northwest is 2 Welbeck Road.  To the southwest are residential properties on the opposite side of Welbeck Road.  A 1.8m solid timber fence and two trees (approximately 4m in height and 8m height) protect the boundary to Welbeck Road.

The proposal is to sub-divide the garden of 53 Cavendish Road, to demolish an existing garage, and to erect a two-storey detached house with integral garage.  The application also proposes to create a new vehicular access for the proposed dwelling.  The two trees fronting Welbeck Road would be retained.  The footprint of the house would measure 10.5m x 9.7m approximately.  The height of the dwelling would be 5.1m to the eaves and 7.3m to the ridge.  The dwelling would be of brick construction and would have a tiled roof.  The doors and windows would be timber.

A 3.2m central section of the front elevation would be recessed from the main facade.  The ground floor would be recessed 0.2m and the first floor would be recessed 1.5m creating a balcony which would be enclosed by decorative metal railings. All habitable room windows are proposed in the front elevation.  Obscure glass block windows and roof lights would provide light to non-habitable rooms and the staircase and hall.

There would be a distance of 3.9m between the gable of the proposed dwelling and the gable of 2 Welbeck Road.  This distance would increase to 5.1m at the rear due to the orientation of the properties.   There are no habitable room windows in the gable wall of 2 Welbeck Road.  There would be a distance of 13.5m to the ground floor extension of 53 Cavendish Road and 17.4m to the main rear elevation.  There would be a distance of 9.1m to the common boundary proposed between these properties.  The distance to the houses on the opposite side of Welbeck Road would be approximately 28m.  The proposed dwelling would be set back 4m from the common boundary with 55 Cavendish Road.

# SITE HISTORY

## None

# CONSULTATIONS

## Director of Environmental Services – no objection, recommendations made for a land contamination study to be conditioned.

PUBLICITY

A site notice was displayed on 24th January 2005.

The following neighbour addresses were notified:

55, 51a Cavendish Road

1, 1a, 1b and 2 Welbeck Road.

REPRESENTATIONS

I have received 6 letters of representation / objection from the occupiers of 5 properties in response to the planning application publicity.  I have also received a letter of objection from the Ellesmere Park Residents Association.  The following issues have been raised:-

- Over development

- Loss of garden space

- Loss of privacy/overlooking

- Overbearing

- Increased problems of parking and the associated dangers.

- Loss of parking for 53 Cavendish Road

- Loss of light/overshadowing

- Odours from kitchen.

- The proposed house would not be in line with the rest of the houses on Welbeck Road.

- The extension approved for 2 Welbeck Road has not been taken into consideration.

- Artificial light from the glass block windows will flood the garden of 55 Cavendish Road when darkness descends.

- Loss of trees

- Ellesmere Park is a conservation area with large gardens The proposal would unduly affect the visual amenity and character of the area .

- The proposal does not accord with the Ellesmere Park Development Control Policy DEV9.

- The use of glass blocks would not be in keeping with the area.

- Work could encroach onto neighbouring land during the construction phase given the proximity to the boundary.

- There are too many building developments in Ellesmere Park which are resulting in contractors vehicles blocking and ruining the roads and verges.

- Increased risk of emergency and service vehicles being unable to gain access

- The proposal is not necessary

# REGIONAL SPATIAL STRATEGY

DP1 – Economy in the use of Land and Buildings

DP3 – Quality in New Development

SD1 - The North West Metropolitan Area

# UNITARY DEVELOPMENT PLAN POLICY

Site specific policies: none

Other policies:	DEV1 - Development Criteria

DEV2 - Good Design

H1 - 	Meeting Housing Needs

T13 - 	Car Parking

EN7 - 	Conservation of Trees and Woodlands

REVISED DEPOSIT DRAFT REPLACEMENT PLAN POLICY

- Site specific policies: none

Other policies: 	DES1 -	 Respecting Context

DES7 -	 Amenity of Users and Neighbours

H1 -	 Provision of New Housing Development

A10 -	 Provision of Car, Cycle and Motorcycle

Parking in New Developments.

PLANNING APPRAISAL

The main planning issues relating to this application are the principle of residential development, the impact of the development on the amenity of the area and neighbouring residents, the amenity provisions for future occupants, and the impact of the development on existing trees.

Principle of residential development

Policy DP1 seeks to ensure that development makes the most efficient use of land.  Policies H1 of the adopted and replacement Unitary Development plans seek to ensure that the City’s housing stock is able to meet the requirements of all groups within Salford by providing a wide range of accommodation.

Policy SD1 of the Regional Spatial Strategy states that development should be focused within the North-West Metropolitan Area, which includes Salford.  Paragraph 5.18 of PPG13 states: ‘the Government is committed to maximising the re-use of previously-developed land and the conversion of existing buildings to promote regeneration and minimise the amount of greenfield land being taken for development’. This approach is reiterated in PPG3 with regard to Housing.  This site falls under the definition of previously developed land, as set out in Annex C of PPG3.  Given that the character of the area is predominantly residential and the site is classed as previously developed land, I am satisfied that the principle of residential development is acceptable.

Impact of the development on the amenity of the area and neighbouring residents

Policy DEV1 requires development to respect its local context and states that regard must be given to a number of factors including the likely scale of traffic generation and the impact of development on sunlight and privacy. Policies DP3 and DEV2 seek to encourage high standards of design.  Policy DES7 states that development will not be permitted where it would have an unacceptable impact on the amenity of the occupiers of other developments.

Council guidance stipulates minimum separation distances that should be maintained between habitable room windows of houses in order to prevent any serious loss of light / prevent overbearing development.  The separation distances that would be maintained between houses are in accordance with this guidance.  Following negotiation, the proposed dwelling has been re-located 4m from the common boundary shared with 55 Cavendish Road.  Given the large size of this neighbours garden I am satisfied that this distance is sufficient to prevent any significant overbearing/overshadowing.  I am also of the opinion that living conditions arising from the alleged artificial light, flooding this garden after darkness descends, would no be unduly armed.

With regard to 2 Welbeck Road, the gables of the two properties would be facing.  The proposed dwelling would not project beyond the rear of 2 Welbeck Road and would project approximately 1.2m beyond the front corner of this neighbour’s property.  The distance between the two properties would be 3.9m at the front.  I consider this relationship to be acceptable and am satisfied that the proposal would not have an overbearing impact on this neighbour or result in a serious loss of light/overshadowing.  However, an extant permission exists for this neighbour’s property for the erection of a two-storey side/rear extension (04/49511/HH – approved December 2004).  If this approval is implemented then a habitable room window for a 5th bedroom would be introduced close to the common boundary with the proposed development.  This window would face Welbeck Road and would be set back 12m from the front elevation of the parent building.  The bedroom which it would serve would measure approximately 3.4m x 2.5m.  Given the small size of the bedroom and given that it is a 5th bedroom already overshadowed by the gable of the parent building (2 Welbeck), I do not consider that the proposal would result in a significant loss of light for the occupiers of 2 Welbeck Road.  There are no habitable room windows proposed in the ground floor element of the approved proposal 04/49511/HH.  I have imposed a condition for all of the window openings in the gable and rear elevations to be fitted with obscure glazing to prevent any loss of privacy for neighbouring residents.

There are a variety of styles of properties within the immediate area.  I consider that the proposed dwelling is adequately designed and is of a scale and massing commensurate with other properties in the area.  Although the dwelling would project 1.2m beyond the nearest corner of 2 Welbeck, I do not consider that it would be at odds with the visual flow of the street and consider the relationship to be acceptable.   I do not consider the proposal represents over development of the site.  I am of the view, therefore, that the development is in accordance with Policies DP3 and DEV2.  I can confirm that the application site is not located within the Ellesmere Park Conservation Area, is not within the area identified in the Ellesmere Park Development Control Policy, and is not allocated under Policy DEV9.

Policy T13 seeks to ensure that adequate and appropriate car parking is provided where necessary.  This is reiterated in Policy A10.  The proposal would provide in-curtilage parking for any future occupiers of the new dwelling.  However, the proposal would result in the loss of in-curtilage parking for the existing dwelling, 53 Cavendish Road.  Supplementary Planning Guidance normally requires the provision of in-curtilage parking for new developments.  I consider that such provision should be made and am satisfied that this is achievable.  A condition has been attached, therefore, for in-curtilage parking to be provided for 53 Cavendish Road.  It is unlikely that the erection one detached dwelling would result in a significant increase in traffic and any associated problems in the area.  I do not consider the proposal would restrict access for emergency and service vehicles.

There is no evidence to suggest work would encroach onto neighbouring land during the construction phase.  Furthermore, the applicant would require the permission of neighbouring occupiers if development were to encroach onto their land.

The necessity of development and the existing building operations which are taking place elsewhere in the local area are not material to this decision.

Amenity Provisions for future occupants

Policy DES7 states that all new development will be required to provide potential users with a satisfactory level of amenity in terms of space, sunlight, daylight, privacy, aspect and layout.  The location of all habitable room windows would be on the front elevations providing aspects that are in accordance with Council Guidance.  I am satisfied that future occupants will be provided with sufficient levels of amenity in terms of light/privacy.  I am also satisfied with the amenity space that would be provided for both the future occupants of the proposed dwelling and 53 Cavendish Road.

Impact on trees

Policy EN7 seeks to protect the City’s treescape.  The two trees to the front of the site offer minimal amenity to the area in my opinion and I would not be opposed to their removal.  However, the plans show that these trees would be retained and Grasscrete is proposed for the driveway which is of a cellular structure which minimises tree root damage.

VALUE ADDED TO DEVELOPMENT

- Pre-application advice was given to help inform the first submission of the application.

- The dwelling has been relocated away from the common boundary with 55 Cavendish Road and the roof height has been lowered and hipped to prevent overbearing/overshadowing development.

- In-curtilage parking has been conditioned for 53 Cavendish Road to minimise on-street parking.

RECOMMENDATION:

Approve Subject to the following Conditions

1.	Standard Condition A01 Five year time limit

2.	No development shall be started until samples of the facing materials to be used for the external elevations and roof of the development have been submitted to and approved in writing by the Local Planning Authority.

3.	The site shall be treated in accordance with a landscape scheme which shall be submitted to and approved by the Local Planning Authority before development is started.  Such scheme shall include full details of trees and shrubs to be planted, walls, fences, boundary and surface treatment and shall be carried out within 12 months of the commencement of development and thereafter shall be maintained to the satisfaction of the Local Planning Authority.  Any trees or shrubs dying within five years of planting shall be replaced to the satisfaction of the Local Planning Authority

4.	Prior to the commencement of the development, the developer shall submit a site investigation report for the approval of the Local Planning Authority.  The investigation shall address the nature, degree and distribution of ground contamination and ground gases on site and shall include an identification and assessment of the risk to receptors as defined under the Environmental Protection Act 1990, Part IIA, focusing primarily on risks to human health and controlled waters.  The investigation shall also address the implications of ground conditions on the health and safety of site workers, on nearby occupied building structures, on services and landscaping schemes and on wider environmental receptors including ecological systems and property.

The sampling and analytical strategy shall be approved by the Local Planning Authority prior to the start of the site investigation survey.  Recommendations and remedial works contained within the approved report shall be implemented by the developer prior to occupation of the site.

Prior to discharge of the Contaminated Land Condition, a Site Completion Report shall be submitted to the Local Planning Authority for approval.  The Site Completion Report shall validate that all works undertaken on site were completed in accordance with those agreed by the Local Planning Authority.

5.	The development permitted by this planning permission shall not be started by the undertaking of a material operation as defined in Section 56(4)(a-d) of the Town and Country Planning Act 1990 until a scheme for a vehicular access and hardstanding measuring at least 2.4m x 4.8m within the remaining curtilage of 53 Cavendish Road has been submitted to and approved in writing by the Local Planning Authority.  The approved scheme shall be completed prior to the commencement of the development hereby approved and shall be retained thereafter.

6.	All windows in the rear and gable elevations shall be fitted with obscure glazing and shall be maintained as such thereafter.

(Reasons)

1.	Standard Reason R000 Section 91

2.	Standard Reason R004A Amenity-area

3.	Standard Reason R004A Amenity-area

4.	Standard Reason R024A Amenity of future residents

5.	Standard Reason R012A Parking only within curtilage

6.	Standard Reason R005A Amenity-neighbours

## Note(s) for Applicant

1.	Please contact United Utilities with regard conecting to the main sewer (0845 050 0108).

2.	Please contact Highway Services (0161 7933795) with regard to the vehicle crossings whichmust be made good at the developers expense.

3.	The developer is advised to contact the Environmental Protection Section of the Environmental Services Directorate with regard to the requirements of the contaminated land condition (0161 7370551)

APPLICATION No:	04/49502/FUL

APPLICANT:	Landfair Ltd

LOCATION:	Land Adjacent To 418 Lower Broughton Road Salford 7

PROPOSAL:	Erection of one pair of semi-detached dwellings

WARD:	Kersal

DESCRIPTION OF SITE AND PROPOSAL

The site is located within the Cliff Conservation Area and is currently occupied by a number of self-seeded trees and shrubs. Surrounding properties are predominantly residential.

The proposed dwellings would have a 13.7m frontage to Lower Broughton Road. They would be 8.9m in length, 6m to the eaves and 9.6m to the ridge. Both properties would have bay windows to the front on both the ground and first floors and dormer windows to the front. They would be a minimum of 6.8m from the back of the footpath. Due to the change in levels between the footpath and the site, steps would be provided with a path to each of the front doors. Car parking would be provided to the rear of the site, accessed from an alley which runs off The Priory.

# SITE HISTORY

In October 2004 an application for the erection of one three storey block comprising six apartments together with alterations to existing vehicular access and associated car parking and landscaping was withdrawn (ref: 04/48921/FUL).

In 1991, an outline planning application for the erection of a three storey building comprising nine flats together with associated car parking and construction of new vehicular and pedestrian accesses was refused for the following reasons: insufficient car parking and resultant impact on highway safety; insufficient space around the building to the detriment of the amenity of future residents; and out of character with the surrounding are due to its density, scale and external appearance (ref: E/28470).

# CONSULTATIONS

## Director of Environmental Services – no objections

PUBLICITY

A site notice was displayed on 24th November 2004

A press notice was published on 25th November 2004

The following neighbour addresses were notified:

13 – 23 Priory Avenue

404, 410 – 420, 431 – 435, 459, 418 Lower Broughton Road

5, 221, Priory Avenue

316 Great Clowes Street

Flats 1 – 5, 418 Lower Broughton Road

23 Grove Lane, Hale, Altrincham

REPRESENTATIONS

I have received four letters of objection to the application. One of the letters has however been sent on behalf of the residents of three properties. The following issues have been raised:

Loss of privacy

Vehicular access and parking

Loss of trees and shrubs

Not in keeping with the character of the conservation area

Reduction in the value of neighbouring properties

Loss of an area of open space

REGIONAL SPATIAL STRATEGY

## Site specific policies: none

Other policies: UR4 – Setting Targets for the Recycling of Land and Buildings

UNITARY DEVELOPMENT PLAN POLICY

Site specific policies: none

Other policies: H1 – Meeting Housing Needs

EN3 – Protected Open Land

DEV2 – Good Design

EN11 – Protection and Enhancement of Conservation Areas

# REVISED DEPOSIT DRAFT REPLACEMENT PLAN POLICY

Site specific policies: CH5/5 – Works Within Conservation Areas (The Cliff)

Other policies: ST11 – Location of New Development

H1 – Provision of New Housing Development

H1A – Managing the Supply of Housing

DES1 – Respecting Context

DES7 – Amenity of Users and Neighbours

PLANNING APPRAISAL

The main planning issues relating to this application are as follows: whether the principle of the proposed development is acceptable; whether the proposed development would enhance or preserve the character and appearance of the Conservation Area; whether there would be an impact on the amenity of neighbouring residents; and whether the application accords with the relevant policies of the Adopted and Revised Deposit Draft Replacement UDP. I will deal with each of the issues in turn below.

Principle of the Proposed Development

Adopted Policy H1 states that the Council will endeavour to ensure that the city’s housing stock is able to meet the housing requirements of all groups within Salford by promoting a number of measures, including the release of land to accommodate new house building.

Adopted Policy EN3 states that the Council will seek to protect and enhance all existing areas of open land. On all land not protected by the Green Belt, planning permission will normally be refused for all development except: where the development is required by or in conformity with the UDP; where the construction of new buildings would be associated with, inter alia, agriculture and forestry and cemeteries; where the development would be associated with the working of minerals; where it would be essential for the protection of an established source of employment; where it would constitute limited infilling which would not adversely affect the character of scale of the existing settlement; or where the development would be essential to the provision and improvement of public services and utilities.

Draft Policy H1 states that new housing development should contribute to the provision of a balanced mix of dwellings within the local area.

Draft Policy H1A states that the release of land for housing will be managed in accordance with the sequential approach set out in Policy ST11.

Draft Policy ST11 states that sites for development will be brought forward in the following order: existing buildings and previously developed land in locations which are or would be well-served by a choice of transport modes; previously-developed land in other locations subject to adequate accessibility; and finally greenfield land in locations which are or would be well-served by a choice of transport modes and are well related to housing, employment, services and infrastructure.

As stated above, the site is currently occupied by shrubs and trees. I have looked into the history of the site and examined historical plans of the area, all of which show the site to be undeveloped. I have also requested the applicant to look into this matter, who has examined maps dating back to the nineteenth century. These too indicate that the site has never been developed. I am therefore of the opinion that the site constitutes greenfield land. There is a general presumption against the development of greenfield sites, as outlined in Planning Policy Guidance Note 3: Housing. This presumption is reflected in the policies of the Revised Deposit Draft Replacement UDP, which also set out the sequential approach to the development of sites. Policy UR4 of RSS also emphasises the priority of the re-use of vacant sites and buildings. Although Adopted Policy EN3 states that limited infilling which would not adversely affect the character or scale of existing settlements could be acceptable on areas of protected open land, this policy pre-dates PPG3 and I am therefore of the opinion that the guidance contains within PPG3 should be afforded greater weight. Greenfield sites in locations which are well-served by a choice of modes of transport and well related to housing, employment, services and infrastructure should only be developed in the absence of existing buildings to convert or previously developed land. There is an abundance of previously developed land in the city which, in accordance with national, regional and local planning policy, should be developed before greenfield sites. In light of the above, I consider the development of this site to be contrary to the thrust of PPG3, Policy UR4 of the RSS and policies H1A and ST11 of the Revised Deposit Draft Replacement UDP.

Impact on the Conservation Area

Adopted Policy EN11 states that in considering applications for development in conservation areas, the Council will assess the extent to which the development is consistent with the desirability of preserving or enhancing the area. High standards of design, which are in keeping with the character of the conservation area, will be encouraged.

Adopted Policy DEV2 states that planning permission for new development will not normally be granted unless the Council is satisfied with the quality of design and the appearance of the development. Regard should be paid to existing buildings, townscape and the character of the surrounding area.

Draft Policy CH5 of the First Deposit Draft Replacement UDP updates Policy EN11 of the Adopted UDP.

Draft Policy DES1 requires developments to respond to their physical context. In assessing the extent to which this has been achieved, regard will be paid to a number of factors, including the relationship to existing buildings and the scale of the proposed development in relation to its surroundings.

The design of the proposed dwellings is similar to that of surrounding residential properties, particularly in terms of the bay and dormer windows to the front. The detailing above and below the windows would also add interest to the proposed dwellings and would again be similar to adjacent houses. In terms of detailed design and contribution to the streetscape, I am of the opinion that the proposed development would enhance the character and appearance of the conservation area. I consider that the proposal accords with the relevant policies of the development plan in with regard to design and the conservation area.

Impact on Residential Amenity

Policy DES7 requires all new developments to provide potential users with a satisfactory level of amenity. Development which would have an unacceptable impact on the amenity of the occupiers or users of other developments will not normally be permitted.

The proposed dwellings would be a minimum of 31m from the properties on the opposite side of Lower Broughton Road. The proposed dwellings would be a minimum of 24m from the rears of the properties on Priory Avenue.  I am satisfied that this is sufficient to ensure that there would be no adverse impact on amenity due to overlooking or loss of privacy. 420 Lower Broughton Road is set considerably further back from the road than the proposed dwelling. There are habitable room windows to the front of No. 420. I am however satisfied with the relationship between the proposed dwellings and No. 420 and do not consider the proposed dwellings would be overbearing or have an unacceptable detrimental impact on the amenity of the occupiers of that property.

I am satisfied that the proposed development would not have an unacceptable detrimental impact on the amenity of neighbouring residents or future residents of the proposed dwelling. The application therefore complies with Draft Policy DES7.

# Other Issues

One of the local residents has objected to the application on the grounds that it would result in a reduction in the value of their property. This is not however a material planning consideration and is not therefore a matter on which I can place any weight in the determination of this application.

A number of local residents are concerned regarding the vehicular access into the site and car parking. Vehicular access into the site would be gained from an alley which runs off Priory Avenue. The application has been amended from that originally submitted and the car parking spaces which were originally located at the top of the alley have been omitted. It is now proposed to provide two car parking spaces within the application site to the rear of the proposed dwellings. I am of the opinion that the proposed amendments have addressed the concerns of local residents in respect of car parking. Given that the application only proposes an additional two dwellings, I do not consider that the scheme would result in a significant increase in traffic in the area and would not have an unacceptable detrimental impact on highway safety. I have no objections to the application in this regard.

CONCLUSION

In conclusion, although I am of the opinion that the design of the dwellings proposed is acceptable and that there would be no unacceptable detrimental impact on the amenity of neighbouring residents or the character or appearance of the Conservation Area, the application site constitutes greenfield land. As such, and given the abundance of previously-developed land in the city, I do not consider the release of such greenfield sites to be acceptable and is not in accordance with the sequential approach set out in national, regional and UDP policy. I am therefore of the opinion that the application is contrary to Draft policies H1A and ST11 and recommend that the application be refused.

RECOMMENDATION:

Refuse For the following Reasons:

(Reasons)

1.	The proposal would result in the development of a greenfield site and would not therefore contribute to achieving the objectives of sustainable development and the prioriy of re-use of vacant sites and buildings, as advocated in Planning Policy Guidance Note 3: Housing and Policy UR4 of the Regional Spatial Strategy. As such, the application is contrary to policies H1A and ST11 of the City of Salford Revised Deposit Draft Replacement Unitary Development Plan.

APPLICATION No:	04/49044/FUL

APPLICANT:	Cara Properties

LOCATION:	Site Of The Former Albert Mill 44 Oldfield Road Salford 5

PROPOSAL:	Demolition of existing building and erection of one 6 storey building comprising 34 apartments, two ground floor commercial units, associated car parking at lower ground/ground floor level and creation of new vehicular access

WARD:	Ordsall

DESCRIPTION OF SITE AND PROPOSAL

This application relates to an existing five-storey mill building situated at the junction of Oldfield Road and James Street close to the junction of Oldfield Road and Chapel Street.  The site drops approximately 1.5m from Oldfield Road to Gibb Street at the rear.  The building is vacant and is on the local list of buildings of archaeological interest as a grade A building.

To the north beyond James Street lies the Salvation Army premises while to the south lies a three storey building that is in poor repair and which is used as a vehicle repair garage.  To the rear beyond Gibb Street lies the listed Islington Mill where permissions have been granted for the conversion of the listed buildings into apartments and workshop/gallery space.

It is proposed to demolish the building and replace it with a six storey building.  The proposed building would be 4.5m higher than the one it would replace.  There would be a total of 34 apartments, 18 with two bedrooms and 16 with one.  Nineteen car parking spaces would be provided at lower ground level accessed from Gibb Street.  There would be two small commercial (A1, A2 or B1) (Retail, Financial and Professional And Business) units on the Oldfield Road frontage.

The design of the replacement building is contemporary but retains the language of the former mill building with the use of a strong brick facade to all elevations and punch holed windows.

# CONSULTATIONS

## British Coal – No objections but provide advice.

Greater Manchester Archaeological Unit -  Albert Mill is shown on the 1848 OS map as ‘Oldfield Road Mill – Silk’. It lies close to the listed Islington Mill and may be roughly contemporary.  As a surviving silk mill of early to mid-19th century origin it is a rare and important industrial site.  It is entered on the Greater Manchester Sites and Monuments Record as 6968 and is also on the Local List of Historic Buildings for Salford.  Every effort should be made to retain the historic fabric of this mill.  In any case, GMAU wish to see an archaeological record made of the mill for archive and research purposes, and to inform conversion works if the mill is saved from demolition.

Archaeological recording should be secured through a planning condition worded as follows (based on PPG16):

‘No development/demolition shall take place within the proposal area until the applicant has secured the scheme of investigation which has been submitted by the applicant and approved by Salford Planning Authority.’

Reason: to make a record of the mill for research and archive purposes.

Director of Environmental Services – recommends that conditions be attached with regard to noise and ground contamination.

Environment Agency – No response to date

Greater Manchester Police Architectural Liaison Unit – No response to date

PUBLICITY

The application has been advertised by means of both site and press notices

The following neighbour addresses were notified:

Dale Garage 44 Oldfield Road

REPRESENTATIONS

I have received one letter of objection in response to the planning application publicity.  The following issues have been raised:-

Business would suffer during and after construction works

# REGIONAL SPATIAL STRATEGY

Site specific policies:	none

Other policies: UR6 Existing Housing Stock and Housing Renewal, SD1 The North west Metropolitan Area – Regional Poles and Surrounding Area, UR4 Setting Targets for the Recycling of Land and Buildings

UNITARY DEVELOPMENT PLAN POLICY

Site specific policies: none

Other policies: H1 Meeting Housing Needs, H6 and H11 Open Space Provision Within New Housing Developments, EN12 Protection and Enhancement of Listed Buildings, T13 Car Parking, DEV1 Development Criteria, DEV2 Good Design

# REVISED DEPOSIT DRAFT REPLACEMENT PLAN POLICY

Site specific policies: none

Other policies: ST11 Location of New Development, H1 Provision of New Housing Development, H8 Open Space provision Within New Housing Development, A10 Provision of Car, Cycle and Motorcycle Parking in New Developments, CH4 Development Affecting the Setting of a Listed Building, DES7 Amenity of Users and Neighbours

PLANNING APPRAISAL

The main planning issues relating to this application are the principle of the development, whether the application accords with policy regarding the setting of listed buildings and whether the concerns of the neighbour should override the wider benefits of the development.

Principle of the Development

Both H1 policies seek to meet the housing needs of all groups within Salford.  Policy H7/1 promotes the improvement of housing in the area.  Policy SD1 states that development should be focused within the North-West Metropolitan Area, which includes Salford.  Policy UR6 seeks to improve the quality of the Region’s housing stock by ensuring sufficient supply across all tenures and values.

The development would see the re-use of an existing building thus complying with criteria 1A of Policy ST11 and the guidance contained within Planning Policy Guidance Note 3 – Housing (PPG3), which seeks to prioritise conversion of buildings and the development of previously developed land over land which has not been previously developed (greenfield land).  This is reiterated in Policy UR4 which states that the redevelopment and re-use of vacant sites within urban areas should be a priority.  This policy sets a target for at least 70% of new dwellings in the region to be on previously developed land.

Policy EN12 states the City Council will not normally permit any development that would be detrimental to the setting of a Listed Building or the environmental quality of the surrounding area.  This is reinforced by draft policy CH4.

The building is vacant and the continued use for manufacture or storage purposes is not practical given the poor state of repair of the building and the fact that the floor to ceiling heights of the existing building would not comply with current height requirements. I am therefore satisfield that the redevelopment of this building is acceptable.  The building is not worthy of Listed status and I am satisfied with the principle of the development.

# Effect on Listed Building

Policies EN12 state that the Council will not normally permit any development that would be detrimental to the setting of a Listed building.

I consider that the appropriate massing and excellent design of the building are such that the setting of Islington Mill would be enhanced by this new development.

The Concerns of the Neighbour

The operator of the garage next door is concerned that both during construction works and after the development is occupied his thus far unhindered use of Gibb Street as an informal extension to his workshop would be restricted.

Gibb Street is a highway and I do not consider that his concerns can be afforded any weight.

Other Issues

Policies T13 and A10 seek to achieve appropriate provision of parking for new developments.  Policy A10 seek to encourage the use of more sustainable forms of travel such as public transport, cycling and car sharing.  Therefore, one of the measures encouraged is maximum levels of parking. The site is located close to a major arterial road that is well served by public transport and within walking distance of the regional centre.  I am therefore satisfied with the level of parking proposed.

Policies H6 & H11 and also H8 require appropriate formal and informal open space within developments.  These policies also require a contribution for open space provision that is outlined in Supplementary Planning Guidance – Provision of Open Space and Recreation Space Associated with New Residential Development.  As such the applicant is required to make adequate provision for open space or contribute through a commuted sum payment to local environmental improvements.  The applicant has agreed to the contribution of £96,755.

VALUE ADDED TO DEVELOPMENT

- pre-application discussions resulted in improvements to the materials and the design

- the development will secure over £96,000 towards the improvement of the local environment.

The principle of the development is acceptable and the proposal will enhance the setting of the adjacent listed building.  I am satisfied that there would be no detrimental effect on any interest of acknowledged importance and I therefore recommend the application for approval.

RECOMMENDATION:

Approve Subject to the following Conditions

1.	Standard Condition A01 Five year time limit

2.	No development shall be started until samples of the facing materials to be used for all external elevations of the development have been submitted to and approved in writing by the Director of Development Services.

3.	Standard Condition F05D Provision of Parking

4.	Standard Condition M05 Site investigation

5.	No development shall commence until a scheme of recycling facilities for the apartments has been submitted to and approved in writing by the Director of Development Services.  Such scheme as is approved shall be implemented in full prior to the occupation of any dwelling.

6.	Prior to commencement of the development; the developer shall undertake an assessment to determine the external noise levels that the proposed residential elements will be subjected to (day time and night time).  The developer shall detail what steps have to be taken to mitigate the disturbance from the above.  The assessment shall have due regard to the Department of the Environment Guidance PPG 24 - Planning and Noise, achieving BS8233: 1999 in all habitable rooms. This assessment and mitigation measures shall be submitted for the approval of Development Services prior to commencement of the development.  Once agreed, all identified noise control measures shall be implemented and thereafter retained.

7.	No development or demolition shall take place until the applicant has secured the implementation of a programme of archaeological work in accordance with a written scheme of investigation that has been submitted to and approved in writing by the Local Planning Authority.

8.	The development permitted by this planning permission shall not be started by the undertaking of a material operation as defined in Section 56(4)(a-d) of the Town and Country Planning Act, 1990 until a Planning Obligation under Section 106 of the Town and Country Planning Act, 1990 has been made and lodged with the Local Planning Authority, and the Local Planning Authority has given its approval in writing. The planning obligation will provide that a commuted sum as required by Policies H6 & H11 of the City of Salford Adopted UDP 1995, H8 of the Revised Deposit Draft Replacement Plan 2003 and SPG7 Provision of Open Space and Recreation Space Associated with New Residential Development and as required by the Chapel Street Planning Obligations Development Control Policy Note will be paid to the Local Planning Authority for open space and recreation space purposes.

(Reasons)

1.	Standard Reason R000 Section 91

2.	Standard Reason R008A Development-Buildings in vicinity

3.	Standard Reason R014A Parking of vehicles - each dwelling

4.	Standard Reason R024A Amenity of future residents

5.	Standard Reason R024A Amenity of future residents

6.	Standard Reason R024A Amenity of future residents

7.	To make a record of the mill for archive and research purposes in accordance with policy EN14 of the City of Salford Unitary Development Plan.

8.	To ensure the residential development provides appropriate open space and recreation space for future occupiers in accordance with policies H6 & H11 of the City of Salford Adopted UDP 1995 and H8 of the Revised Deposit Draft Replacement Plan 2003 and appropriate local environmental improvements in accordance with the Chapel Street Planning Obligations Development Control Policy Note.

## Note(s) for Applicant

1.	The proposed development lies within a coal mining area. In the circumstances the Applicant should take account of any coal mining related hazards to the stability of their proposal. Developers must also seek permission from the Coal Authority before undertaking any operation that involves entry into any coal or mines of coal, including coal mine shafts and adits and the implementation of site investigations or other works.   Property specific summary information on any past, current or proposed surface and underground mining activity to affect the development can be obtained from the Coal Authority. The Coal Authority Mining Reports Service can be contacted on 0845 762 6848 or at www.coal.gov.uk

2.	The applicant is advised that the requirements of all the conditions precedent must be satisfied prior to the commencement of the development. Failure to satisfy the conditions precedent renders all development unauthorised and unlawful and appropriate action may be taken by the Council.

3.	The decision to grant planning permission has been taken having regard to the policies and proposals in the City of Salford Unitary Development Plan set out below, and to all relevant material considerations, including Supplementary Planning Guidance.  There are no other material planning considerations that outweigh this finding: DEV1 Development Criteria, H6 and H11 Open Space Provision Within New Housing Developments, EN12 Protection and Enhancement of Listed Buildings, DEV2 Good Design.

APPLICATION No:	04/49793/FUL

APPLICANT:	Easter Developments (Salford) Limited

LOCATION:	Centenary Park Centenary Way Eccles

PROPOSAL:	Erection of six light industrial units (B1, B2 and B8 Uses) with ancillary offices, services yards and car parking together with creation of new vehicular access (Phase 1)

WARD:	Weaste And Seedley

DESCRIPTION OF SITE AND PROPOSAL

This rectangular site, measuring 370 metres by 150 metres, is on land to the north of Centenary Way Eccles. The site is to the north of the Coronet Flour Mill which itself is adjacent to the Manchester Ship Canal. Other surrounding uses are; industrial units to the east, vacant land to the west and the Ladywell Park and Ride site to the north which is above a steep embankment. Centenary Way finishes approximately half the way down this site.

Planning permission is sought for the erection of six units providing 17,611 square metres of commercial floorspace. The application seeks permission for light industrial, general industrial or distribution uses within the units. Vehicular access for unit one is proposed to be taken from a spur off the existing roundabout on Centenary Way. Vehicular access for units two to six would be taken directly from Centenary Way. The application includes an extension to Centenary Way. Landscaping and associated car parking are also proposed around each of the units.

Unit one is to the west of the site and is orientated toward the existing roundabout on Centenary Way. All other units are sited to the east of unit one. The majority of parking is positioned between the units and Centenary Way with a five metre wide landscape zone proposed between the road and the car parking areas. The units have a ridge height level of between 10 metres and 14 metres. The ancillary offices are positioned within each of the units to face toward Centenary Way.

# SITE HISTORY

No relevant previous planning history.

# CONSULTATIONS

## Director of Environmental Services – Concerns over the impact of night time noise upon the nearby Weaste Quarry site and recommend condition regarding site investigations.

Greater Manchester Police Architectural Liaison Unit – Concerns over height of railings

Greater Manchester Geological Unit – Comments received explaining advice should be taken from the Director of Environmental Services

Health and Safety Executive – Do not advise against granting planning permission

United Utilities – Provide advice on water supply. Informative attached.

Environment Agency – No objections recommend conditions.

Strategic Director of Housing and Planning – No objection to the principle of development, concerns over the external appearance of all the units and position of landscaping screen along the highway, no objection to the level of parking.

PUBLICITY

A press notice was published on 20th January 2005

A site notice was displayed on 13th January 2005

The following neighbour addresses were notified:

Coronet Flour Mill, Centenary Way

651, 659, 665 –669 (all) Eccles New Road

REPRESENTATIONS

I have received no letters of representation / objection in response to the planning application publicity.

# REGIONAL SPATIAL STRATEGY

## DP3 Quality in New Developments

UNITARY DEVELOPMENT PLAN POLICY

Site specific policies: EC13/1 Sites for Industry and Warehousing, T16/6  Major Road Schemes

Other policies: DEV1 Development Criteria, DEV2 Good Design, DEV4 Design and Crime

# REVISED DEPOSIT DRAFT REPLACEMENT PLAN POLICY

Site specific policies: E3/14 Sites for Employment Development, A9/2 Provision of New Highways (Broadway Link)

Other policies: DES1 Respecting Context, DES11 Design and Crime

PLANNING APPRAISAL

The main planning issues relating to this application are the allocation for employment, the Broadway link, visual impact of the development upon the locality, access and parking, secured from crime, ground conditions and noise impacts upon nearby sites.

The site is allocated for industry and warehousing in the adopted plan and also for offices or light industry or general industry or storage and distribution in the revised deposit draft plan. I am therefore satisfied that the proposal comprises acceptable land uses. Within both UDP’s there is a requirement for the road link between Centenary Way and Broadway to be completed. Part of the red line plan covers the route of this link and the applicant proposes to construct part of the road link as part of his development. Otherwise I am satisfied that the route of the link will not be affected by this development. I consider the proposal complies with the area based policies.

Each unit has been sited toward the rear of the site, backing onto the slope that rises northwards toward the Ladywell Park and Ride site. Loading bays are positioned towards the rear of units and would not be visible from Centenary Way. The appearance of the units are generally acceptable however unit one which fronts the existing roundabout would benefit from an improvement to the front elevation. The applicant has agreed to be bound by a condition requiring submission of further details of this elevation. Between the units and the road are parking areas for each unit with a landscaped 5 metre wide buffer strip screening much of the site from Centenary Way. Subject to the above condition regarding the front elevation of unit 1 and details of boundary treatment and samples of materials I am satisfied with the appearance of the development.

The submitted layout plan shows pedestrian and cycle routes alongside the vehicle highway for the part of the highway the applicant intends to construct. I am satisfied with the detail of this and also of the vehicular entrances into the site. The level of on site parking is acceptable with regard to current parking standards however I intend to impose a condition requiring details of disabled parking which should be located within the parking of each unit closest to the buildings entrance. I also propose a condition for details of motorcycle and bicycle parking. The developer will need to enter into a S278 agreement to undertake works to the highway.

Greater Manchester Police have raised concerns over the height of railings which are proposed at 1.8m high. I have recommended a condition for railing details. The applicant proposes 24 hour use and as the offices face toward the entrances to the site I consider there is natural surveillance provided.

The Director of Environmental Services has proposed a condition regarding ground investigation which I have attached. I have also received comments regarding the noise impact of this development upon the Weaste Quarry site (not allocated for housing), which as Members will be aware has two current residential planning applications that are being considered. A panel date has not been set for the residential applications. The siting of the apartments is 130 metres away from the loading bay of the nearest industrial unit. The applicant and land owner (Peel) have stated that a restriction in hours of operation will detrimentally affect viability on the application site. Peel also own the site of Weaste Quarry and have confirmed that any residential amenity concerns on that site can be resolved through a re-design or improved sound insulation. Given the above and as this site is allocated for employment I do not propose to attach a condition limiting hours of operation.

VALUE ADDED TO DEVELOPMENT

I have negotiated improvements to landscaping, parking and external appearance of the site. The scheme has also been subject of pre-application discussions.

# CONCLUSION

The proposed scheme would provide suitable employment uses in accordance with the land use allocation in both the adopted and deposit draft UDP’s. The proposal would not disrupt the route of the Broadway link and would include the construction of part of that link. I am satisfied with the visual impact of the development subject to appropriate conditions outlined above. I have no highway objections and recommend approval subject to the attached conditions.

RECOMMENDATION:

Approve Subject to the following Conditions

1.	Standard Condition A01 Five year time limit

2.	The landscape scheme hereby approved shall be carried out within 12 months of the commencement of development and thereafter shall be maintained to the satisfaction of the Local Planning Authority.  Any trees or shrubs dying within five years of planting shall be replaced to the satisfaction of the Director of Development Services.

3.	No development shall be started until samples of the facing materials to be used for the walls and roofs of the development have been submitted to and approved in writing by the Local Planning Authority.

4.	No development shall commence until provision has been made for Centenary Way to accommodate a right hand turn lane in accordance with details that have first been submitted to and agreed in writing by the Local Planning Authority.  The approved right hand turn lane shall be installed before the 'Broadway Link' (as indicated by adopted UDP policy T16/6) has been implemented and made open to traffic.

5.	This permission shall relate to the amended plans (04-96-02 and 04-96-03) received on 14 the February 2005 which shows the amended landscape proposals.

6.	Notwithstanding the boundary treatment details shown on the submitted plans, full details of the design, height and colour treatment of the boundary treatments shall be submitted to and approved by the Local Planning Authority before development is started.  The development shall proceed in accordance with the approved details.

7.	Notwithstanding that the submitted drawings are acceptable in terms of the design ethos, height, scale and massing of the building, no development shall commence on site until such time as the detailed design of the elevations of unit number one have been submitted to, and approved in writing by the local planning authority.

8.	Prior to the commencement of the development, the developer shall submit a site investigation report for the approval of the Local Planning Authority.  The investigation shall address the nature, degree and distribution of ground contamination and ground gases on site and shall include an identification and assessment of the risk to receptors as defined under the Environmental Protection Act 1990, Part IIA, focusing primarily on risks to human health and controlled waters.  The investigation shall also address the implications of ground conditions on the health and safety of site workers, on nearby occupied building structures, on services and landscaping schemes and on wider environmental receptors including ecological systems and property.

The sampling and analytical strategy shall be approved by the Local Planning Authority prior to the start of the site investigation survey.  Recommendations and remedial works contained within the approved report shall be implemented by the developer prior to occupation of the site.

Prior to discharge of the Contaminated Land Condition, a Site Completion Report shall be submitted to the Local Planning Authority for approval.  The Site Completion Report shall validate that all works undertaken on site were completed in accordance with those agreed by the Local Planning Authority.

9.	Notwithstanding the disabled parking spaces shown on the submitted plans, full details of the number and locations of disabled parking spaces associated with each unit shall be submitted to and approved in writing by the Local Planning Authority before development is started.  Such approved parking facilities shall be made available prior to the first use of the premises, to the satisfaction of the Local Planning Authority.

10.	No development shall be started until full details of the location, design and construction of bicycle parking facilities have been submitted to and approved in writing by the Local Planning Authority.  Such approved bicycle parking facilities shall thereafter be constructed and made available for use before the first occupation of the building.

(Reasons)

1.	Standard Reason R000 Section 91

2.	Standard Reason R004A Amenity-area

3.	Standard Reason R004A Amenity-area

4.	Standard Reason R015A Safety-users of highway

5.	Standard Reason R004A Amenity-area

6.	Standard Reason R004A Amenity-area

7.	Standard Reason R004A Amenity-area

8.	Standard Reason R028A Public safety

9.	To ensure that adequate car parking provision is made for disabled people within the curtilage of the site in accordance with policy DEV5 of the City of Salford Unitary Development Plan.

10.	To ensure that adequate provision is made for the parking of bicycles within the curtilage of the site in accordance with policy DEV1 of the City of Salford Unitary Development Plan.

## Note(s) for Applicant

1.	The proposed development lies within a coal mining area. In the circumstances the Applicant should take account of any coal mining related hazards to the stability of their proposal. Developers must also seek permission from the Coal Authority before undertaking any operation that involves entry into any coal or mines of coal, including coal mine shafts and adits and the implementation of site investigations or other works.   Property specific summary information on any past, current or proposed surface and underground mining activity to affect the development can be obtained from the Coal Authority. The Coal Authority Mining Reports Service can be contacted on 0845 762 6848 or at www.coal.gov.uk

2.	The applicant's attention is drawn to the contents of the attached letter from United Utilities.

3.	The applicant's attention is drawn to the contents of the attached letter from the Director of Environmental Services.

4.	The applicant's attention is drawn to the contents of the attached letter from The Environment Agency.

5.	The applicant's attention is drawn to the contents of the attached letter from the Greater Manchester Police Architectural Liaison Unit.

APPLICATION No:	04/49744/REM

APPLICANT:	Shenton Homes

LOCATION:	402/412 Liverpool Road Eccles

PROPOSAL:	Details of the design and external appearance of one 3 storey building comprising 18 flats together with associated landscaping, car parking and construction of new vehicular access

WARD:	Winton

DESCRIPTION OF SITE AND PROPOSAL

This application relates to a row of six derelict shops located on the A57 (Liverpool Road).  The application site covers an area of approximately 900m2 and measures 30m x 30m.  To the north (on Atherton Street and Atherton Way) and to the south-east (on the opposite side of Liverpool Road) are two storey residential properties.  To the north-east of the site (on the opposite side of Atherton Street) on Liverpool Road are shops, some with residential accommodation at first floor level.  To the south-east is an area of grassed land approximately 20m x 30m which includes several trees approximately 10m in height.  Beyond this land is the Peel Green Key Local Centre.

Outline permission was approved in August 2004 for the siting and means of access for the erection of one three storey block of 18 apartments (04/48346/OUT).  This application considers the remaining reserved matters (design, external appearance and landscaping).

Through the use of modern materials and a flat roof design, the three-storey building would have a contemporary feel.  The walls would be constructed from red/brown facing brickwork, with two three-storey bays projecting from the front elevation and one three-storey central bay projecting from the rear elevation which would be finished in blue/grey render.   A buff Art Stone string course is proposed between ground and first floor levels.  Dark Grey polyester powder coated (PPC) aluminium would form the roof eaves and the window frames.  The main entrance would be located centrally in the front elevation and would also be constructed from PPC aluminium.  Buff Art Stone is proposed around the entrance to increase its prominence. To the front and side elevations a series of Juliet balconies are proposed which would have a ‘grid-like’ design.

To the rear of the building elements of the upper floors would project over areas of ground floor parking.  Cylindrical steel columns would support these upper floors.  The height of the building would be 9.3m.  As expressed in the outline application, all habitable room windows would be located in the front and side elevations with none in the rear elevation.

Landscaping is proposed to the front of the building within an area of defensible space approximately 2.5m in depth.  Additional landscaped areas are proposed adjacent to the parking in the northern and western corners of the site (measuring 15m X 0.6m and 7m X 2m respectively).

# SITE HISTORY

In August 2004, outline permission was approved for the siting and means of access to one 3-storey block of 18 apartments (04/48346/OUT).

# CONSULTATIONS

## Director of Environmental Services – no objection, recommendations made.

Environment Agency – no objection.

## Architectural Liaison Officer – concerns were raised over the entrances and advice was given for the boundary walls and vehicular access.  Amendments have been made since receiving these comments.

## Strategic Director of Planning and Housing – advice offered with regard to housing provision in the Eccles ward.

PUBLICITY

A press notice was published on 13th January 2005..

A site notice was displayed on 5th January 2005.

The following neighbour addresses were notified:

377-393(odd), 398, 400, 422-438(even), 386-396 (even), Alexandra House, Liverpool Road

2-8 (even), 14 Atherton Street

1-11Atherton Way

REPRESENTATIONS

I have received 1 letter of objection in response to the planning application publicity.  The following issues have been raised:-

- Concern is raised over the security of the scheme.  The neighbouring resident expresses concern for the future safety of the development and makes reference to the possibility of arson attacks.

REGIONAL SPATIAL STRATEGY

## DP3 Quality in New Development

UNITARY DEVELOPMENT PLAN POLICY

Site specific policies: 	none

Other policies: 	DEV1 Development Criteria

DEV2 Good Design

DEV4 Design and Crime

# REVISED DEPOSIT DRAFT REPLACEMENT PLAN POLICY

Site specific policies: 	none

Other policies: 	DES1 Respecting Context

DES7 Amenity of Users and Neighbours

DES11 Design and Crime

PLANNING APPRAISAL

The main planning issues relating to this application are the design and massing of the proposal and its impact on the amenity of neighbouring residents, and the landscaping proposed.

Design and massing of the proposal and the impact on the neighbouring residents

Policy DP3 requires development to demonstrate good design and respect for its setting.  Policies DEV1, DEV2, DES1 and DES7 require development to respect its context regarding height, scale, mass, materials and also privacy and sunlight/daylight of surrounding properties.  Although the building is three-storey, with neighbouring properties being predominantly two-storey, the flat roof design ensures that the building respects the scale and massing of the existing properties.

Council Guidance stipulates minimum separation distances from habitable room windows in order to provide future occupants and neighbouring residents with sufficient levels of amenity.  At outline stage, the applicant indicated that all habitable room windows would be located in the front and side elevations to prevent overlooking to the properties at the rear on Atherton Way.  This proposal adheres to that principle and so I am satisfied that the proposal would not have a detrimental impact on the amenity of neighbouring residents.

I consider the design of the proposal to be acceptable.  The proposal would offer a modern addition to the area which would add interest and variation to the street scene.  The materials proposed for the development would be of a high quality and so I am satisfied with the design and external appearance of the development.

Policies DEV4 and DES11 require development to be designed to minimise the risk of crime.  The proposal has been amended to take account of the Greater Manchester Police Architectural Liaison Officer’s comments, the details of which have been forwarded to the applicant.  Some of the issues raised, such as the boundary wall details, would be dealt with via condition.  I am satisfied that the proposal is in accordance with the provisions of policies DEV4 and DES11.

The planting schedule proposed for the landscaping scheme is acceptable.  However, no details of the boundary walls have been provided for consideration.  A condition has been attached for the necessary details to be provided.

VALUE ADDED TO DEVELOPMENT

- Pre-application discussions were held to agree the issues and details of the proposal prior to its submission.

- The side entrance to apartment 1 has been amended to improve security for the development.

- Details of the planting schedule were secured for consideration.

# CONCLUSION

The main planning issues relating to this application are the design and massing of the proposal and its impact on the amenity of neighbouring residents, and the landscaping proposed.  I am satisfied with the design and massing of the proposal and consider that it will not have a detrimental impact on the amenity of neighbouring residents.  The landscaping proposed is acceptable.  The proposal is in accordance with the provisions of the Development Plan and all material considerations have been taken into account.

RECOMMENDATION:

Approve Subject to the following Conditions

1.	The landscape scheme as detailed on drawing No. 1394/01 hereby approved shall be carried out within 12 months of the substantial completion of the development and shall be retained thereafter.  Any trees or shrubs dying within five years of planting shall be replaced to the satisfaction of the Local Planning Authority.

2.	Prior to the commencement of development details of all boundary walls/fences shall be submitted to and approved in writing by the Local Planning Authority.  The agreed boundary details shall form part of the landscaping scheme referred to in condition 1 and shall be carried out within 12 months of the substantial completion of the development and shall be retained thereafter.

3.	No development shall be started until samples of the facing materials to be used for the external elevations of the development have been submitted to and approved in writing by the Local Planning Authority.

4.	Prior to any part of the development hereby approved first becoming occupied, the car parking layout shown on drawing No. P2105-101 Revision D shall be marked out and hardsurfaced and made available thereafter at all times for the parking or residents and visitors associated with the development.

(Reasons)

1.	Standard Reason R004A Amenity-area

2.	Standard Reason R004A Amenity-area

3.	Standard Reason R004A Amenity-area

4.	Standard Reason R012A Parking only within curtilage

## Note(s) for Applicant

1.	The developer should contact the City's Highway Services on 0161 7933795 with regards to the proposed footway crossing.

2.	Please be aware that the Prunus x hillieri 'Spire' is planted on a non-suckering rootstock.  In addition the City Council would like to promote sustainability and encourage and environmentally friendly approach for the landscaping scheme proposed:

1.  There is a requirement for granular slow release fertiliser for the planting beds and it is suggested that pelleted chicken manure or other similar material is used.

2.  Instead of importing topsoil, which is full of weed seeds and possibly Japenese Knotweed or other undesirable species, recycled compost, now readily available in Manchester should be used.

3.  Instead of rubber tree ties, which do not degrade and will damage the trees if not regularly checked, the use of hessian as a tree tie is better, since it will rot away after a couple of years.

4.  Subject to approval by the City's Drainage Section, the use of a permeable surface for the car park and drainage towards the plant beds or even installation of a soakaway would mean that less water would go to the storm sewer and therefore contribute to the alleviation of flash flooding.

APPLICATION No:	04/49784/OUT

APPLICANT:	Salvation Land Development

LOCATION:	Land On South Side Of Holyoake Road Walkden Worsley

PROPOSAL:	Outline planning application for residential development together with alteration to existing vehicular access

WARD:	Walkden South

DESCRIPTION OF SITE AND PROPOSAL

This application relates to existing industrial and garage premises on Holyoake Road.

The site covers an area of 1.8 hectares and is bounded by the rear of terraced houses on Walkden Road to the west, by two and three storey residential development to the north beyond Holyoake Road, by residential development to the east and by railway lines and Walkden station to the south.

The site is generally flat but is approximately 2.5m lower than Holyoake Road with the surrounding land generally falling from north to south.  The railway lies on a slight embankment above the site.

The application has been submitted in outline and it is proposed to erect a total of 141 two and three storey residential units on the site.  The application had originally sought permission for the means of access and siting of the buildings but approval for siting is now not being sought at this stage as a result of my concerns regarding the layout.

The applicant has submitted a noise report, tree survey, industrial land supply statement and planning statement in support of the application.

# CONSULTATIONS

## Director of Environmental Services – no objection, recommendations made.

Environment Agency – No objection, recommendations made.

Greater Manchester Police Architectural Liaison Unit – No objections, but provides advice.

Network Rail – No comments to date

Strategic Director of Planning and Housing – makes a number of comments as follows:-

Planning Policy considerations

The site is unallocated in the Adopted Unitary Development Plan and the Revised Deposit Draft UDP. However, the following range of issues needs to be addressed, having regard to various non-site specific policies in both versions of the Plan: -

- Sequential Approach to Development

- Loss of Employment Land

- Mix of Dwellings / Density Issues

- Parking provision

- Provision of Open Space

- Design

The resultant re-use of the land is consistent with Regional Planning Guidance for the North West (now RSS), which supports “the recycling of land and buildings” (RPG13, paragraph 5.1) as a key means of delivering the urban renaissance required within the Region. The applicant should submit information to demonstrate that the proposal is acceptable in terms of Adopted Policy EC3 and Revised Deposit Draft UDP Policy E5, in order to assist in determining whether the principle of residential use is acceptable at the site. The proposed residential use should reduce general disturbance at the site to the benefit of local residential amenity that may result from the use of the existing industrial premises. I would prefer the applicant to provide more houses and to reduce the proposed provision of apartments at the site in order to encourage the retention of families in the city and to reduce the potential contribution to an oversupply of apartments across the city, which would be contrary to Adopted Policy H1. In the event that the proposal is considered acceptable I advise the applicant to comply with the open space requirements and to submit a design statement.

# Director of Strategy And Regeneration  - comments received

PUBLICITY

The application has been advertised by means of both site and press notices.

The following neighbour addresses were notified:

70 to 108 Walkden Road

1, 27 to 41 and 50 to 60 Holyoake Road

Flats 1 to 18 Coniston House Holyoake Road

Flats 1 to 21 Rydal House Holyoake Road

1 to 4 Blantyre Avenue

1 to 4 Holly Avenue

2 to 6 Chilham Road

1 to 5 (incl) Old Oake Close

REPRESENTATIONS

I have received 3 letters of  objection and one letter in support in response to the planning application publicity.  The following issues have been raised:-

- The bollards on Holyoake road must not be removed

- Insufficient school places

# REGIONAL SPATIAL STRATEGY

Site specific policies: None

Other policies: SD1 - The North West Metropolitan Area – Regional Poles and   Surrounding Areas

- DP1 – Economy in the Use of Land and Buildings

UNITARY DEVELOPMENT PLAN POLICY

Site specific policies: None.

Other policies:	H1 Meeting Housing Needs

H6 & H11 Open Space Provision

DEV1 Development Criteria

EC3 – Reuse of Sites and Premises

# REVISED DEPOSIT DRAFT REPLACEMENT PLAN POLICY

Site specific policies:	None

Other policies:	ST11 Location of New Development

H1 Provision of New Housing Development

H8 Open Space Provision Associated With New Housing Development

DES1 Respecting Context

E5 Development Within Established Employment Areas

DES7 Amenity of Users and Neighbours

PLANNING APPRAISAL

The main planning issues relating to this application is whether the principal of residential development is acceptable.

# Principle of development

Policy SD1 of the Regional Spatial Strategy states that development should be focused within the North-West Metropolitan Area, which includes Salford.  The development would see the re-use of brownfield land thus complying with criteria 1b of Policy ST11 and the guidance contained within Planning Policy Guidance Note 3 – Housing (PPG3), which seeks to prioritise the development of such land over land that has not been previously developed (greenfield land).

Policy EC3 states that, where existing non-retail commercial premises become vacant, the city council will seek to re-use or redevelop them for similar uses except where one or more of three criteria are satisfied.  The first criterion states that development must not result in a material or unacceptable shortfall in the range of sites available for economic development and the third states that alternative development is acceptable where there is a strong environmental case for rationalising land uses.   Draft policy E5 states that planning permission will only be granted for the re-use or redevelopment of sites within an established employment area for non-employment uses where the development would not compromise the operating conditions of other remaining employment uses and where one or more of a number of criteria apply.  Those criteria relevant in this instance are where the developer can clearly demonstrate that there is no current or likely future demand for the site or building for employment purposes or where there is a strong environmental case for rationalising land uses.    Development of this land would result in the loss of a relatively small commercial site located in an otherwise residential area. I therefore consider that the proposed development satisfies both relevant criteria of both policy E3 and draft policy E5.

In 2004 the City Council commissioned a study into employment provision across the City.  This recommended that the Council should protect and maintain the City’s existing allocated employments sites to maintain an adequate supply of employment land, but as this application site is not part of a wider industrial/employment area, I am satisfied that its loss to residential purposes would not have a significant detrimental impact upon the city’s overall employment land supply.  Furthermore, the applicant has demonstrated that existing occupiers of the site have required a move to larger premises and that the long term attractiveness of this isolated site to employment uses is limited.  I consider that therefore the re-use of the site for residential purposes is further justified.

The surrounding uses are predominantly residential, therefore I am satisfied that the proposal would respect the character of the area.  Although the railway is located to the immediate south of the site the noise report undertaken on behalf of the developer shows that there would be no detrimental effect on any dwellings on this site.  As siting is not being considered however at this stage it is appropriate to attach a condition requiring the submission of a further noise report once siting has been determined.

Other Issues

The application has been amended as a result of both my and the Strategic  Director of Planning and Housing’s concerns regarding siting.  I am satisfied that the numbers of dwellings proposed on this site are appropriate.

The bollards on Holyoake Road will be retained and the issue of school places is one that the Director of Education takes into account as a result of building commencements and it is not normally an issue that is considered at this stage.

CONCLUSION

The main planning issues relating to this application are whether the principal of residential development is acceptable.  I am satisfied that, in principle, residential development is acceptable for this site and that the site can appropriately be developed for the number of dwellings specified.  The proposal is in accordance with the Adopted and Revised Unitary Development Plans and so I recommend approval.

RECOMMENDATION:

Approve Subject to the following Conditions

1.	Standard Condition A02 Outline

2.	No development shall be started until full details of the following reserved matters have been submitted to and approved by the Local Planning Authority:

- the layout of the site, including the disposition of buildings and roads and provision for parking and servicing;

- plans and elevations showing the design of all buildings and other structures;

- the colour and type of facing materials to be used for all external walls and roofs;

- a landscape scheme for the site which shall include details of trees and shrubs to be planted, any existing trees to be retained, or felled indicating the spread of the branches and trunk positions, walls, fences, boundary and surface treatment.

3.	This permission shall not relate to the siting details shown on the submitted plan, nor does it imply that permission is likely to be forthcoming for such details.

4.	The development permitted by this planning permission shall not be started by the undertaking of a material operation as defined in Section 56(4)(a-d) of the Town and Country Planning Act, 1990 until a Planning Obligation under Section 106 of the Town and Country Planning Act, 1990 has been made and lodged with the Local Planning Authority, and the Local Planning Authority has given its approval in writing. The planning obligation will provide that an area of open space and childrens equipped play space is provided within the site or a commuted sum as required by Policies H6 & H11 of the City of Salford Adopted UDP 1995, H8 of the Revised Deposit Draft Replacement Plan 2003 and SPG7 Provision of Open Space and Recreation Space Associated with New Residential Development will be paid to the Local Planning Authority for open space and recreation space purposes.

5.	Prior to the commencement of the development, the developer shall submit a site investigation report for the approval of the Local Planning Authority.  The investigation shall address the nature, degree and distribution of ground contamination and ground gases on site and shall include an identification and assessment of the risk to receptors as defined under the Environmental Protection Act 1990, Part IIA, focusing primarily on risks to human health and controlled waters.  The investigation shall also address the implications of ground conditions on the health and safety of site workers, on nearby occupied building structures, on services and landscaping schemes and on wider environmental receptors including ecological systems and property.

The sampling and analytical strategy shall be approved by the Director of Environmental Services prior to the start of the site investigation survey.  Recommendations and remedial works contained within the approved report shall be implemented by the developer prior to occupation of the site.

Prior to discharge of the Contaminated Land Condition, a Site Completion Report shall be submitted to the Local Planning Authority for approval.  The Site Completion Report shall validate that all works undertaken on site were completed in accordance with those agreed by the Local Planning Authority.

6.	Prior to the commencement of the development, the developer shall undertake an assessment to determine the external noise levels from the adjacent railway line that these proposed residential elements will be subjected to (day time and night time). The assessment shall have due regard to the Department of Environment Guidance Note PPG 24 - Planning and Noise. The assessment shall be submitted for approval of Development Services prior to the commencement of the development. Once agreed, all noise control measures shall be implemented and thereafter retained.

7.	No development shall commence until a scheme for the improvement/making up of Mullineux Street has been submitted to and approved in writing by the Local Planning Authority.  Such scheme as is approved shall be implemented in full proir to the occupation of any unit.

(Reasons)

1.	Standard Reason R001 Section 92

2.	Standard Reason R002 Reserved Matters

3.	Approval for siting was removed from the application by the applicant  prior to the application being considered.

4.	To provide for the provision of open space in accordance with policies H6 and H11 of the City of Salford Unitary Development Plann the interests of future residents.

5.	Standard Reason R024A Amenity of future residents

6.	Standard Reason R024A Amenity of future residents

7.	Standard Reason R004A Amenity-area

## Note(s) for Applicant

1.	The proposed development lies within a coal mining area. In the circumstances the Applicant should take account of any coal mining related hazards to the stability of their proposal. Developers must also seek permission from the Coal Authority before undertaking any operation that involves entry into any coal or mines of coal, including coal mine shafts and adits and the implementation of site investigations or other works.   Property specific summary information on any past, current or proposed surface and underground mining activity to affect the development can be obtained from the Coal Authority. The Coal Authority Mining Reports Service can be contacted on 0845 762 6848 or at www.coal.gov.uk